| Job Hazard Analysis |  |  |
| --- | --- | --- |
| Organization: I&E | Division: WASTE WATER TREATMENT FAC. | Section/Shop: WWTF |
| Task: RAKING A BAR SCREEN |  |  |
| Conducted By: | Reviewed By: | Approved By: G.G. MURPHY/TACTICAL SAFETY SPEC./BASE SAFETY |
| Date:20130611 | Date:20130611 | Date:20130611 |
| Sequence of Steps | Potential Accidents or Hazards | Preventative Measures |
| - TURN VENTILATION ON TO RID CONFINED SPACE OF GASES AND ENSURE THAT PERMIT TO ENTER CONFINED SPACE IS OBTAINED. - OBSERVATION OF AREA (TO INCLUDE VENTILATION SYSTEM AND AROUND WORK AREA) - DON PERSONAL PROTECTIVE EQUIPMENT (P.P.E.) AND ENSURE THERE IS AN ATTENDANT AND ENTRY PERSON PRESENT. - TEST THE ATMOSPHERE WITHIN THE CONFINED SPACE. - RADIO CONTACT BASE OPS. TO INFORM ENTRY INTO CONFINED SPACE. - SET UP HOIST AFTER INSPECTION SYSTEM (FALL ARREST, FALL PROTECTION EQUIPMENT, ETC…) - ENTER AFTER ATMOSPHERE HAS BEEN CONFIRMED CLEARED AND RAKE THE BAR SCREEN. - FILL BUCKETS WITH WASTE AND ATTACH TO RAISING DEVICE FOR THE ATTENDANT TO LIFT TO SURFACE. - EXIT CONFINED SPACE WITH ALL EQUIPMENT. - RADIO BASE OPS. TO INFORM THAT PROCESS IS COMPLETE AND CONFINED SPACE HAS BEEN SECURED. | - 1. ELECTRICAL HAZARDS WHEN TURNING VENTILATION SYSTEM ON, HEARING LOSS DUE TO NOISE PRODUCED FROM VENTILATION SYSTEM. - 2. RODENTS, VERMENTS, AND PESTS (BLACK WIDOWS, SNAKES, ETC…; SLIPS, TRIPS, AND FALLS; ELECTRICAL HAZARDS. - 3. INCORRECT P.P.E., UNSERVICEABLE P.P.E., OR LACK OF UNDERSTANDING ON HOW TO DON/DOFF P.P.E. - 4. ATMOSPHERE MONITOR MALFUNCTIONS OR IS NOT CALIBRATED. - 5. ENGULFMENT HAZARDS, RESPIRATORY HAZARDS, CHEMICAL/BIO-HAZARDOUS MATERIAL (SPLASHING). - 6. HOIST FAILS, FALL ARREST SYSTEM FAILS TO FUNCTION, FALL INTO PIT OR THROUGH HATCH AT SURFACE LEVEL. - 7. SLIPS, TRIPS, AND FALLS (LADDER SLIPPERY OR DAMP, FALL INTO PIT). - 8. BIO-HAZARDOUS WASTE CAN SPLASH ON EMPLOYEES, AND ERGONOMIC ISSUES FROM LIFTING IMPROPERLY, RESPIRATORY HAZARDS. - 9. SLIPS, TRIPS, AND FALLS WHEN EXITING PIT, ERGONOMIC ISSUES FROM LIFTING EUIPMENT AND GEAR FROM CONFINED SPACE, AND RESPIRATORY HAZARDS. ATTENDANT COULD FALL INTO OPEN PIT. 10. PINCH HAZARDS CLOSING CONFINED SPACE HATCH; SLIPS, TRIPS, AND FALLS; RESPIRATORY HAZARDS, ELECTRICAL HAZARDS, ENGULFMENT IN SPACE WHEN EXITING. | 1. TRAINING (ARC FLASH AND ELECTRICAL SAFETY, FALL PROTECTION, CONFINED SPACE, ETC…) AND ON THE JOB TRAINING CONDUCTED BY EXPERINCED EMPLOYEES (SHIFT OPS. TRAIN NEW EMPLOYEES). 2. INSPECT WORKPLACE PRIOR TO WORK STARTING, ENSURE TO INSPECT SMALL CREVICES. 3. INSPECT P.P.E. PRIOR TO START OF WORK EACH DAY FOR SERVICEABILITY (STEEL TOE BOOTS, TYVEX SUITS, SAFETY GOGGLES, OIL AND SLIP RESISTANT SOLES ON BOOTS). 4. VERIFY CALIBRATION AND TEST MACHINE BEFORE LEAVING WORK CENTER. TEST AT ALL LEVELS IN SPACE TO ENSURE ALL LEVELS ARE FREE OF GASES. 5. ENSURE S.O.P. WRITTEN OUTLINING REQUIRED STEPS FOR ENTERING CONFINED SPACES, CONFINED SPACE TRAINING. 6. INSPECT FOR WEAR AND DAMAGE, UTILIZE A FALL ARRESTER AT ALL TIMES, ENSURE ATTENDANT IS WEARING FALL PROTECTION GEAR WHILE OVER OPEN HATCH OF PIT. 7. TRAINING, P.P.E., COMMUNICATION SYSTEM ESTABLISHED FOR EMPLOYYE IN PIT AND ATTENDANT OUTSIDE OF PIT. 8. ERGONOMICS TRAINING, UTILIZE LIFTING DEVICE VICE MANUAL LIFT TO SURFACE. 99. PROPER DONNING/DOFFING P.P.E. CLASSES, FALL PROTECTION TRAINING. 10. WEAR GLOVES THAT WILL MINIMIZE PINCH HAZARDS WHEN CLOSING HATCH TO PITS, ENSURE S.O.P. IS IN PLACE FOR EXITING PITS. |